21st Century AV Ltd

RISK ASSESSMENT & METHOD STATEMENT

21CQ28863 – London Borough Of Southwark  |  London Borough Of Southwark

PROJECT DETAILS

| Project Reference | 21CQ28863 |
| --- | --- |
| Project Name | 21CQ28863 – London Borough Of Southwark |
| Client | London Borough Of Southwark |
| Site Address | Queens Road 3, 1 Lugard Road, Peckham, SE15 2HP London |
| Contractor | 21st Century AV Ltd |
| Works Description |  |
| Start Date | TBC |
| Expected Duration | TBC |
| Document Status | For Review |
| Date | March 2026 |

Quoted Works Summary

| SKU / Product Code | Qty | Description |
| --- | --- | --- |
|  | 1 | Samsung QB65C Smart Signage Display |
|  | 1 | Flat Screen Articulating Wall Mount |
|  | 1 | SY 4 Button Wall Control Panel |
|  | 1 | RS232 Cable |
|  | 3 | Rally Bar Mini & Tap IP Bundle |
|  | 3 | Logitech Wall Mount For Video Bars Camera mount wall mountable, cart mountable for Rally Bar |
|  | 3 | Mersive Solstice Pod Gen3 SGE Enterprise |
|  | 3 | Solstice Pod Gen3 Power Supply |
|  | 3 | SY 6 button Keypad Controller with rotary volume control. In White |
|  | 3 | Logitech Tap Scheduler Purpose-Built Scheduling Panel for Meeting Rooms |
|  | 6 | 1m High Speed HDMI Male-Male 4k@60Hz 4:4:4 Cable |
|  | 3 | 2-Gang Dry Lining Box, 47mm (D) |
|  | 6 | 2m Cat.6A S/FTP LSZH Network Cable, Grey |
|  | 3 | 7.5m High Speed HDMI Male-Male 4k@60Hz 4:2:0 Cable |
|  | 3 | 10m CAT6a/RJ45 S/FTP LSZH Network Cable, Grey |
|  | 2 | Samsung QB85C Smart Signage Display PLA4 Parallel Action Dual arm for screens up to 100" |
|  | 1 | Logitech Rally Plus System - PTZ Camera, 2 x Rally Microphones & 2 Rally Speakers |
|  | 1 | Logitech Tap Cat5e |
|  | 1 | Thinksmart Core MTR For Logitech |
|  | 2 | Logitech Rally Mic Pod |
|  | 1 | Logitech - Network coupler for Logitech Rally; Rally Bar, Bar Mini, Mic Pod |
|  | 4 | Logitech Mic Pod Mount |
|  | 1 | Logitech Rally Mic Pod Hub for the Rally Ultra-HD ConferenceCam, Graphite, Desk, Logitech, Rally, Rally Plus |
|  | 1 | Logitech Tap Table Mount |
|  | 1 | Logitech Scribe |
|  | 1 | 4x4 HDBaseT™ CSC AV Matrix - 70m (4K60Hz 4:4:4 up to 40m), ARC, Bi-directional IR, RS-232, PoC, Simultaneous HDMI / HDBaseT™ on output 1, video Smart Scaling on HDBaseT outputs, Independent Audio Matrix, Line Level Volume Control, RS-232 & TCP/IP Control, HDCP 2.2 compliant, Web-GUI |
|  | 1 | HDBaseT™ Receiver supporting 4K 60Hz 4:4:4 to 40m (1080p up to 70m), RS-232 Pass through, Bi-directional IR, Bi-directional PoC, HDCP 2.2 |
|  | 1 | HDBaseT™ CSC Transmiiter supporting 4K 60Hz 4:4:4 to 40m (1080p up to 70m), Bi-directional IR & RS-232, EDID Management, HDMI Loop-Out, Audio Breakout, Bi-directional PoC, HDCP 2.2 |
|  | 3 | 7.5m CAT6a/RJ45 S/FTP LSZH Network Cable, Grey |
|  | 1 | 15m CAT6a/RJ45 S/FTP LSZH Network Cable, Grey |
|  | 2 | 1m USB 3.2 Type A Cable |
|  | 1 | Samsung QB85C Smart Signage Display |
|  | 1 | Twin Sound Bar and Video Conference Mount (Compatible up to 110" screens) |
|  | 1 | Samsung QB75C Smart Signage Display |
|  | 1 | Parallel Action Dual arm for screens up to 100" |
|  | 1 | Logitech Rally Plus Video Conference Equipment |
|  | 1 | 4x2 HDBaseT™ CSC AV Matrix Kit - 70m (4K60Hz 4:4:4 up to 40m), ARC, Bi-directional IR, RS-232, PoC, Simultaneous HDMI / HDBaseT™ on output 1, video Smart Scaling on HDBaseT outputs, Auto Source Selection, Audio Breakout, RS-232 & TCP/IP Control, HDCP 2.2 compliant, Web-GUI. Supplied with 2x RX70CS HDBaseT™ Receivers |
|  | 4 | LG 65" LG65UA751C LED Smart TV |
|  | 4 | Tilting Wall mount for Screens up to 80" |
|  | 7 | SCHURTER IEC C13 Socket, 10mm Cable Entry |
|  | 7 | SCHURTER Rewireable IEC C14 Plug ( E), Straight |
|  | 6 | PRO ELEC 6 Gang Surge Protected Extension Lead with USB, 5m, Black |
|  | 2 | PRO ELEC UK Mains Plug with 13A Fuse, Black (Box of 20) |
|  | 3 | CONCORDIA TECHNOLOGIES Cable Tie Base, Black, 28x28mm, Pack of 100 |
|  | 14 | PRO POWER Cable Ties Black 200mm x 2.5mm 100 Pack |
|  | 1 | PRO POWER Expandable Braided Sleeving, 20mm Dia., 100m, Black |
|  | 4 | PRO POWER Expandable Braided Sleeving, 10mm Dia., 10m, Black |
|  | 14 | PRO SIGNAL High Speed 4K UHD Slim HDMI Lead with Ethernet, 2m Black |
|  | 7 | PRO SIGNAL High Speed 4K UHD Slim HDMI Lead with Ethernet, 1m Black |
|  | 2 | PRO SIGNAL 4K Premium High Speed HDMI Lead, 3m |
|  | 11 | AV:LINK 1 Gang Black Brush Plate, White |
|  | 1 | SPEEDY RJ45 RJ45 Cat6 Plugs, Pack of 100 |
|  | 10 | SPEEDY RJ45 Speedy RJ45 Cat6 Strain Relief Boots, Pack of 10 |
|  | 10 | Logitech Rally Mic Pod white |
|  | 1 | Logitech Mic Pod Hub |
|  | 10 | Logitech Ceiling Mount for Microphone - White |
|  | 3 | Logitech Rally Mounting Kit 12 of 14 |
|  | 1 | Hardware Delivery |
|  | 1 | BDT DELIVERY OF KIT |
|  | 3 | Cables & Consumables - Upto 6 Person Room |
|  | 3 | Cables & Consumables - 6+ Person Room |
|  | 1 | Site Survey (incl Parking/Travel/Congestion) |
|  | 1 | Project Drawing & Inc Tech Check per room |
|  | 4 | Elevation Drawings - 30 Mins Per Room |
|  | 1 | Enhanced Connection Drawing |
|  | 1 | RAMS - Detailed Risk & Method Statement |
|  | 2 | First Fix Cabling - 21st Engineering AV Team In-Hours Mon-Fri 9am - 5pm |
|  | 7 | 21st Engineering AV Team In-Hours Mon-Friday 9am - 5pm |
|  | 5 | Rubbish Removal - WEEE Waste - per pallet / pallet space (1m2 x 1.5m high / 250kg each) |
|  | 1 | Hazardous Waste Consignment Charge |
|  | 1 | Installation Commissioning |
|  | 1 | Configuration of the hardware - full access to clients IT Team will be required for the duration of the configuration. |
|  | 1 | Project Handover - Demonstration (not training) of all rooms, systems check and Project Signoff |
|  | 1 | SY Wall Panel Programming - per unique panel (offsite) |
|  | 2 | SY Wall Panel Programming to additional panels (per panel same programme) |
|  | 9 | Travel per vehicle/team per day - In London (within M25) |
|  | 1 | Project Management On Site - Per Day Includes Site Attendances, Trade Liason, Contract Administration |
|  | 4 | Project Management Off Site - General Project Delivery Mon-Fri up to 7.5 hours Includes Trade Liason, Contract Administration |
|  | 1 | Product Training |
|  | 6 | 4 Gang Surge Protected Extension Lead, White 2m - |
|  | 1 | O&M Manual |

DOCUMENT AUTHORISATION

| Role | Name | Title | Signature | Date |
| --- | --- | --- | --- | --- |
| Document Author |  |  |  |  |
| Authorised By |  |  |  |  |
| Authorised By (Client) |  |  |  |  |

EMERGENCY CONTACTS

| Contact | Tel | Mobile | Role |
| --- | --- | --- | --- |

APPLICABLE UK LEGISLATION & REGULATIONS

| Applicable UK Legislation |  |
| --- | --- |
| Health & Safety at Work Act 1974 | Management of H&S at Work Regs 1999 |
| Manual Handling Operations Regs 1992 | Work at Height Regulations 2005 |
| PUWER 1998 | COSHH 2002 |
| Control of Noise at Work Regs 2005 | PPE at Work Regulations 2022 |
| CDM Regulations 2015 | Electricity at Work Regulations 1989 |
| Control of Asbestos Regulations 2012 | RIDDOR 2013 |

RISK RATING SYSTEM

| Likelihood | Score | Severity | Risk = Likelihood × Severity |
| --- | --- | --- | --- |
| Highly Unlikely | 1 | Trivial | 1 | No Action Required (1) |
| Unlikely | 2 | Minor Injury | 2 | Low Priority (2–6) |
| Possible | 3 | Over 3-Day Injury | 3 | Medium Priority (7–9) |
| Probable | 4 | Major Injury | 4 | High Priority (10–14) |
| Certain | 5 | Incapacity or Death | 5 | Urgent Action Required (≥15) |

| PPE Required for this Project |  |  |  |  |
| --- | --- | --- | --- | --- |
| Safety Boots (steel toe cap) | Hi-Visibility Vest | Safety Glasses | Latex / Nitrile Gloves | Hard Hat |
| Dust Mask (FFP2) | Hearing Protection |  |  |  |

Persons at Risk

✓ 21CAV Staff

| Hazard | Consequence | Pre-Control Risk |  |  | Control Measures | Post-Control Risk |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L | S | Risk |  | L | S | Risk |
| Manual Handling |  | 3 | 3 | 9 | 1. Use mechanical aids (sack trucks, lifting trolleys) for items over 20 kg. 2. Team lift required for screens and equipment over 40" — minimum two persons. 3. Pre-plan the route and clear all access paths before moving equipment. 4. Wear appropriate gloves and safety footwear at all times. 5. Conduct a task-specific manual handling assessment prior to lifting. 6. Take regular breaks to avoid fatigue during prolonged lifting tasks. | 2 | 2 | 4 |
| Slips, Trips & Falls (Same Level) |  | 3 | 3 | 9 | 1. Keep all work areas clean and tidy — return tools to bags after each task. 2. Secure trailing cables immediately with cable covers or gaffer tape. 3. Erect hazard warning signs and barriers around active work zones. 4. Ensure adequate lighting in all areas where work is taking place. 5. Wear steel-toe-cap safety footwear at all times on site. 6. Report and isolate any spills or wet surfaces immediately. | 2 | 2 | 4 |
| Working at Height |  | 3 | 3 | 9 | 1. Select appropriate access equipment: podium steps, kick stools, or access tower. 2. Inspect all access equipment before each use — do not use damaged items. 3. Set up access equipment on stable, level ground only. 4. Maintain three points of contact when ascending or descending ladders. 5. Erect barriers and warning signs below any overhead work. 6. Wear a hard hat within the overhead works exclusion zone. 7. Never over-reach — reposition access equipment as required. 8. Do not use chairs, boxes or improvised platforms as working platforms. | 2 | 2 | 4 |
| Electrical Hazards |  | 3 | 3 | 9 | 1. All electrical work to be carried out by competent, authorised persons only. 2. Isolate and lock off power circuits before making any electrical connections. 3. Test for dead using an approved voltage indicator before touching any conductors. 4. Do not use damaged cables, plugs or extension leads — remove from service immediately. 5. All temporary power supplies to use RCD protection. 6. Comply with BS 7671 (IET Wiring Regulations) 18th Edition at all times. 7. Notify the client facilities team before isolating any shared power circuits. | 2 | 2 | 4 |
| Struck by Falling Objects |  | 3 | 3 | 9 | 1. Use tool lanyards for all hand tools when working at height. 2. Establish a clearly marked exclusion zone below all overhead work. 3. Verify load-bearing capacity of all fixings before mounting AV equipment. 4. Hard hats to be worn by all persons within the exclusion zone. 5. Use safety straps on displays during installation until fully secured. 6. Never leave partially fixed equipment unattended during installation. | 2 | 2 | 4 |
| Dust & Debris (Including Drilling) |  | 3 | 3 | 9 | 1. Check the asbestos register or obtain an asbestos survey before any drilling. 2. Use dust extraction equipment when drilling into walls, floors or ceilings. 3. Wear FFP2 or FFP3 dust masks during all drilling and cutting operations. 4. Wear safety glasses/goggles during drilling and cutting. 5. Seal off the work area from occupied spaces using temporary screens or sheeting. 6. Dispose of all waste and debris in accordance with site waste procedures. | 2 | 2 | 4 |
| Hidden Services (Electrical, Plumbing, Gas) |  | 3 | 3 | 9 | 1. Use a cable and pipe detector (CAT & Genny) before every drilling operation. 2. Obtain up-to-date services drawings from the client facilities team prior to works. 3. Mark all detected services clearly before any drilling commences. 4. If uncertain, do not drill — seek written confirmation from the client. 5. Ensure first aid and emergency contact numbers are known before commencing. | 2 | 2 | 4 |
| Sharps & Hand / Power Tools |  | 3 | 3 | 9 | 1. Wear cut-resistant gloves when handling raw cable ends and sheet metal. 2. Inspect all hand tools before use — do not use damaged or worn tools. 3. Power tools must be PAT tested and inspection tags in date. 4. Keep blade guards fitted on all cutting tools when not in active use. 5. First aid kit to be on site and its location communicated to all operatives. 6. Report and record all cuts, lacerations and near-misses. | 2 | 2 | 4 |
| Lone Working |  | 3 | 3 | 9 | 1. Lone working must be pre-approved by the 21CAV project manager. 2. Lone worker to check in with a nominated contact at start and end of every session. 3. Use a lone worker app or buddy check-in system for isolated work areas. 4. Mobile phone must be fully charged with network coverage confirmed. 5. Emergency procedures to be communicated before commencing lone work. 6. No lone working at height or with high-risk power tools. | 2 | 2 | 4 |

METHOD STATEMENT — SEQUENCE OF WORKS

1.  1. Pre-Start Checks

•  Attend site induction at Queens Road 3, 1 Lugard Road, Peckham, SE15 2HP and sign in with the principal contractor or site manager before any works commence.

•  Confirm all relevant areas including the AV rack location, video conferencing rooms, ceiling mounting zones, and structured cabling routes are accessible and handed over to the AV team.

•  Verify that all drawings, AV design schematics, rack elevations, and cable schedules have been reviewed and are on site or available digitally for reference.

•  Inspect all areas where ceiling works and overhead mounting will take place, confirming ceiling type, load-bearing capacity, and the presence of any hidden services above the ceiling void.

•  Confirm all deliveries of AV equipment, cabling, and consumables are scheduled and that a secure on-site storage area has been designated for equipment protection.

2.  2. Delivery and Materials Handling

•  Receive all AV equipment deliveries at the designated loading area, checking each item against the delivery notes and project equipment schedule before bringing items into the building.

•  Inspect displays, screens, video conferencing units, audio equipment, and rack infrastructure for any transit damage and report and record any defects immediately before installation.

•  Transport all equipment to the storage area using appropriate trolleys and manual handling techniques, ensuring heavy rack enclosures and displays are handled by a minimum of two operatives.

•  Store all sensitive AV equipment, including control system processors, audio amplifiers, and video conferencing codecs, in a clean, dry, and secure area away from other trades.

•  Ensure all structured cabling materials including cable drums, patch panels, trunking, and conduit are staged in proximity to their installation zones to minimise double handling.

3.  3. Access Equipment Setup

•  Erect mobile elevated work platforms or scaffold towers in areas where ceiling works, overhead cable routing, and display mounting at height are required, ensuring all equipment is inspected and tagged before use.

•  Confirm that all operatives using access equipment hold a valid PASMA or IPAF certificate relevant to the access equipment being used on site.

•  Establish exclusion zones beneath all overhead working areas using barrier tape and appropriate signage to prevent unauthorised access during ceiling works and overhead mounting activities.

•  Position access equipment on stable, level ground and ensure all brakes, outriggers, and stabilisers are engaged before any operative ascends to work at height.

4.  4. Installation Works

•  Install all structured cabling routes including containment, trunking, conduit, and cable trays throughout the building, following the approved cable routing drawings prior to pulling any cables.

•  Pull and route all AV, data, and power cables through the agreed containment routes, ensuring all cables are correctly labelled at each end in accordance with the cable schedule.

•  Carry out all ceiling works including the fixing of ceiling mounts, projector drop poles, speaker brackets, and any bespoke steelwork required for overhead display and AV equipment mounting.

•  Build and dress the AV rack in accordance with the rack elevation drawings, installing all equipment including control system processors, AV matrix switchers, amplifiers, and patch panels in the correct order.

•  Install all displays, screens, and video conferencing camera and codec units at their designated locations, securing all wall and ceiling mounts to structural substrates using manufacturer-specified fixings.

5.  5. Cable Termination and Testing

•  Terminate all structured cabling connections including RJ45 patch panels, keystone outlets, and any fibre optic terminations in accordance with the approved cable schedule and relevant TIA or ISO standards.

•  Test all structured cabling links using a certified cable tester, recording results for each run and ensuring all links meet the required category performance specification for the project.

•  Terminate all AV signal cables including HDMI, HDBaseT, audio, and control cabling at both the rack end and device end, cross-referencing all terminations against the as-installed cable schedule.

•  Carry out point-to-point signal continuity checks on all AV and control cabling before powering up any equipment, confirming correct polarity and pinout throughout.

•  Power up the AV rack in a controlled sequence, checking each piece of equipment for correct operation before proceeding to control system programming and system-level commissioning.

6.  6. Final Checks and Handover

•  Commission the control system by loading the approved program, testing all control macros, display switching, audio presets, and video conferencing call functions to confirm full system operation as designed.

•  Conduct end-to-end system testing of all AV zones including audio system playback, display image quality, video conferencing camera and microphone performance, and content routing through the AV matrix.

•  Remove all waste materials, packaging, cable offcuts, and access equipment from site, leaving all work areas clean and clear in accordance with the site rules.

•  Compile and hand over all project documentation including as-installed drawings, cable test results, equipment warranties, control system source code, and operational user guides to the client representative.

•  Carry out a formal client walkthrough and demonstration of all installed AV systems, obtaining a signed acceptance certificate to confirm the works have been completed to the client's satisfaction.

| Operative Sign-Off |  |  |
| --- | --- | --- |
| I have read and understood this Risk Assessment and Method Statement and agree to comply with its requirements. |  |  |
| Print Name | Signature | Date |

| Document Control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rev | Date | Prepared By | Checked By | Description |
| 01 | 23/03/2026 | 21st Century AV Ltd | — | Initial Issue |