21st Century AV Ltd

RISK ASSESSMENT & METHOD STATEMENT

21CQ30103 – Osprey Install  |  Osprey Install

PROJECT DETAILS

| Project Reference | 21CQ30103 |
| --- | --- |
| Project Name | 21CQ30103 – Osprey Install |
| Client | Osprey Install |
| Site Address | Osprey House, 3 The Bullpens, Herriard, Basingstoke, RG25 2PH Hampshire |
| Contractor | 21st Century AV Ltd |
| Works Description |  |
| Start Date | TBC |
| Expected Duration | TBC |
| Document Status | For Review |
| Date | March 2026 |

Quoted Works Summary

| SKU / Product Code | Qty | Description |
| --- | --- | --- |
|  | 1 | Logitech Rallybar - All-in-One Conferencing Device |
|  | 1 | Logitech Rally, Expansion Mic Pod |
|  | 1 | Mic Pod Desk Bracket - requires drilling a small hole into the desk |
|  | 1 | Logitech Cat Coupler - Mic Pod cable extension |
|  | 1 | Extron - Desk Mounted Cable Cubby (1 UK A/C Power socket and Pass through for 2x USB Cables) |
|  | 1 | Lindy 8m USB 3.2 Active Extension Pro Type A |
|  | 1 | USB 3.0 A-to-C 2m |
|  | 1 | Kramer 10m HDMI - 7.6m Grey 1080p@60Hz 12 bit HDMI Male-Male Cable |
|  | 1 | 10m CAT6a/RJ45 S/FTP LSZH Network Cable, Grey |
|  | 1 | 1m High Speed HDMI Male-Male 4k@60Hz 4:4:4 Cable |
|  | 4 | 1m Length of Surface Trunking White (TBC at SSV) |
|  | 1 | Rubber Floor Matting for Cable Run between Desk and Display Wall - Black, 1.8m |
|  | 1 | Hardware Delivery |
|  | 1 | Site Survey (incl Parking/Travel) |
|  | 1 | 21st Engineering AV Team In-Hours Mon-Friday 9am - 5pm |
|  | 1 | Cables & Consumables - Extenders, connectors, cable ties etc.. |
|  | 1 | Travel per vehicle/team per day - Outside M25 |
|  | 1 | Configuration of the hardware - full access to clients IT Team will be required for the duration of the configuration. |

DOCUMENT AUTHORISATION

| Role | Name | Title | Signature | Date |
| --- | --- | --- | --- | --- |
| Document Author |  |  |  |  |
| Authorised By |  |  |  |  |
| Authorised By (Client) |  |  |  |  |

EMERGENCY CONTACTS

| Contact | Tel | Mobile | Role |
| --- | --- | --- | --- |

APPLICABLE UK LEGISLATION & REGULATIONS

| Applicable UK Legislation |  |
| --- | --- |
| Health & Safety at Work Act 1974 | Management of H&S at Work Regs 1999 |
| Manual Handling Operations Regs 1992 | Work at Height Regulations 2005 |
| PUWER 1998 | COSHH 2002 |
| Control of Noise at Work Regs 2005 | PPE at Work Regulations 2022 |
| CDM Regulations 2015 | Electricity at Work Regulations 1989 |
| Control of Asbestos Regulations 2012 | RIDDOR 2013 |

RISK RATING SYSTEM

| Likelihood | Score | Severity | Risk = Likelihood × Severity |
| --- | --- | --- | --- |
| Highly Unlikely | 1 | Trivial | 1 | No Action Required (1) |
| Unlikely | 2 | Minor Injury | 2 | Low Priority (2–6) |
| Possible | 3 | Over 3-Day Injury | 3 | Medium Priority (7–9) |
| Probable | 4 | Major Injury | 4 | High Priority (10–14) |
| Certain | 5 | Incapacity or Death | 5 | Urgent Action Required (≥15) |

| PPE Required for this Project |  |  |  |  |
| --- | --- | --- | --- | --- |
| Safety Boots (steel toe cap) | Hi-Visibility Vest | Safety Glasses | Latex / Nitrile Gloves | Hard Hat |
| Dust Mask (FFP2) | Hearing Protection |  |  |  |

Persons at Risk

✓ 21CAV Staff

| Hazard | Persons at Risk | L | S | Risk | Control Measures | L | S | Risk |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Manual Handling | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Use mechanical aids (sack trucks, lifting trolleys) for items over 20 kg. 2. Team lift required for screens and equipment over 40" — minimum two persons. 3. Pre-plan the route and clear all access paths before moving equipment. 4. Wear appropriate gloves and safety footwear at all times. 5. Conduct a task-specific manual handling assessment prior to lifting. 6. Take regular breaks to avoid fatigue during prolonged lifting tasks. | 2 | 2 | 4 |
| Slips, Trips & Falls (Same Level) | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Keep all work areas clean and tidy — return tools to bags after each task. 2. Secure trailing cables immediately with cable covers or gaffer tape. 3. Erect hazard warning signs and barriers around active work zones. 4. Ensure adequate lighting in all areas where work is taking place. 5. Wear steel-toe-cap safety footwear at all times on site. 6. Report and isolate any spills or wet surfaces immediately. | 2 | 2 | 4 |
| Working at Height | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Select appropriate access equipment: podium steps, kick stools, or access tower. 2. Inspect all access equipment before each use — do not use damaged items. 3. Set up access equipment on stable, level ground only. 4. Maintain three points of contact when ascending or descending ladders. 5. Erect barriers and warning signs below any overhead work. 6. Wear a hard hat within the overhead works exclusion zone. 7. Never over-reach — reposition access equipment as required. 8. Do not use chairs, boxes or improvised platforms as working platforms. | 2 | 2 | 4 |
| Electrical Hazards | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. All electrical work to be carried out by competent, authorised persons only. 2. Isolate and lock off power circuits before making any electrical connections. 3. Test for dead using an approved voltage indicator before touching any conductors. 4. Do not use damaged cables, plugs or extension leads — remove from service immediately. 5. All temporary power supplies to use RCD protection. 6. Comply with BS 7671 (IET Wiring Regulations) 18th Edition at all times. 7. Notify the client facilities team before isolating any shared power circuits. | 2 | 2 | 4 |
| Struck by Falling Objects | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Use tool lanyards for all hand tools when working at height. 2. Establish a clearly marked exclusion zone below all overhead work. 3. Verify load-bearing capacity of all fixings before mounting AV equipment. 4. Hard hats to be worn by all persons within the exclusion zone. 5. Use safety straps on displays during installation until fully secured. 6. Never leave partially fixed equipment unattended during installation. | 2 | 2 | 4 |
| Dust & Debris (Including Drilling) | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Check the asbestos register or obtain an asbestos survey before any drilling. 2. Use dust extraction equipment when drilling into walls, floors or ceilings. 3. Wear FFP2 or FFP3 dust masks during all drilling and cutting operations. 4. Wear safety glasses/goggles during drilling and cutting. 5. Seal off the work area from occupied spaces using temporary screens or sheeting. 6. Dispose of all waste and debris in accordance with site waste procedures. | 2 | 2 | 4 |
| Hidden Services (Electrical, Plumbing, Gas) | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Use a cable and pipe detector (CAT & Genny) before every drilling operation. 2. Obtain up-to-date services drawings from the client facilities team prior to works. 3. Mark all detected services clearly before any drilling commences. 4. If uncertain, do not drill — seek written confirmation from the client. 5. Ensure first aid and emergency contact numbers are known before commencing. | 2 | 2 | 4 |
| Sharps & Hand / Power Tools | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Wear cut-resistant gloves when handling raw cable ends and sheet metal. 2. Inspect all hand tools before use — do not use damaged or worn tools. 3. Power tools must be PAT tested and inspection tags in date. 4. Keep blade guards fitted on all cutting tools when not in active use. 5. First aid kit to be on site and its location communicated to all operatives. 6. Report and record all cuts, lacerations and near-misses. | 2 | 2 | 4 |
| Lone Working | • 21CAV Staff • Client Staff • Others | 3 | 3 | 9 | 1. Lone working must be pre-approved by the 21CAV project manager. 2. Lone worker to check in with a nominated contact at start and end of every session. 3. Use a lone worker app or buddy check-in system for isolated work areas. 4. Mobile phone must be fully charged with network coverage confirmed. 5. Emergency procedures to be communicated before commencing lone work. 6. No lone working at height or with high-risk power tools. | 2 | 2 | 4 |

METHOD STATEMENT — SEQUENCE OF WORKS

1.  1. Pre-Start Checks

•  Confirm site access arrangements with the client contact at Osprey House, 3 The Bullpens, Herriard, Basingstoke, RG25 2PH prior to mobilisation.

•  Review and sign off the RAMS documentation with all attending AV installation engineers before works commence on site.

•  Conduct a walk-round survey of all rooms designated for video conferencing, audio system, display installation, and AV rack locations to verify dimensions, power availability, and structural suitability.

•  Confirm that all structured cabling routes, containment pathways, and ceiling void or wall access points are clear and suitable for cable management works.

•  Verify that all equipment scheduled for installation, including video conferencing codecs, audio DSP, control system processors, displays, and rack furniture, has been received and matches the project specification.

2.  2. Delivery and Materials Handling

•  Co-ordinate delivery of all AV equipment, rack furniture, cabling materials, and associated fixings to Osprey House, ensuring the unloading area is clear and safe.

•  Inspect all delivered items for transit damage prior to signing delivery notes and report any discrepancies or damage to the project manager immediately.

•  Transport equipment and materials to designated working areas within the building using appropriate manual handling techniques, using trolleys or mechanical aids for heavy rack furniture and display screens.

•  Store all equipment securely in a designated on-site area, ensuring fragile items such as display screens and video conferencing cameras are kept upright and protected from damage.

3.  3. Access Equipment Setup

•  Identify all areas requiring work at height, including display mounting positions, ceiling cable routes, and high-level rack installations, and select appropriate access equipment such as stepladders or podium steps.

•  Inspect all access equipment before use to confirm it is in serviceable condition, free from defect, and rated for the combined weight of the operative and any tools or materials being carried.

•  Position and secure access equipment on level, stable floor surfaces at each work location and ensure exclusion zones are established beneath any overhead working areas.

•  Ensure that operatives working at height have been briefed on the requirements of the Work at Height Regulations 2005 and that no lone working takes place when using access equipment.

4.  4. Installation Works

•  Install AV rack furniture in the designated equipment location, level and secure the rack to the floor or wall as required, and begin populating the rack with patch panels, power distribution units, AV processors, video conferencing codecs, audio DSP units, and control system processors in accordance with the rack elevation drawings.

•  Install structured cabling containment, including trunking and conduit, along agreed routes throughout the building to serve video conferencing rooms, audio system zones, and display locations, then pull all category and signal cables through the containment runs leaving adequate service loops at each termination point.

•  Mount all display screens and video conferencing cameras at the specified positions using appropriate wall or ceiling brackets, verifying fixing integrity into solid structural elements and ensuring correct viewing height and angle.

•  Install all audio system components, including loudspeakers, amplifiers, and microphone systems, at the positions defined in the project drawings, securing all fixings appropriately and routing speaker cabling back to the AV rack through the installed containment.

•  Connect the control system processor within the rack and install any room-based control touchpanels or button panels at designated positions, routing control cabling back to the rack via the installed cable management pathways.

5.  5. Cable Termination and Testing

•  Terminate all structured cabling at patch panels and outlet faceplates in accordance with T568B wiring standards, then carry out continuity and performance testing on every run using a certified cable tester, recording all results.

•  Terminate all audio, video, and control signal cables within the AV rack and at peripheral devices, dress and label all cables clearly within the rack using consistent labelling that matches the as-built documentation.

•  Power on the AV rack equipment in a controlled sequence and verify that all devices initialise correctly with no fault indications before proceeding to system integration.

•  Load the control system programme onto the control processor and conduct initial testing of all control pathways, confirming that the system correctly operates the video conferencing codec, audio DSP, displays, and any room switching functions.

•  Test the video conferencing system end-to-end by placing test calls to confirm audio, video, and content sharing functionality, and test the audio system across all zones to verify correct coverage, levels, and routing through the DSP.

6.  6. Final Checks and Handover

•  Conduct a full system walkthrough with the client representative at Osprey House to demonstrate the operation of the video conferencing, audio, control, and display systems and confirm all scoped functionality is working as specified.

•  Carry out a thorough site clean-up, removing all off-cuts, packaging, and installation debris from all working areas and leaving the site in a clean and tidy condition.

•  Compile and hand over the complete project documentation package to the client, including as-built drawings, cable test results, equipment manuals, warranty information, and control system programme files.

•  Provide a user training session for the designated client personnel, covering day-to-day operation of the video conferencing system, audio system, and control interfaces.

•  Obtain a client sign-off on the project completion certificate confirming satisfactory delivery of all scoped works including video conferencing, audio installation, AV rack build, structured cabling, control systems, display installation, and commissioning.

| Operative Sign-Off |  |  |
| --- | --- | --- |
| I have read and understood this Risk Assessment and Method Statement and agree to comply with its requirements. |  |  |
| Print Name | Signature | Date |

| Document Control |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rev | Date | Prepared By | Checked By | Description |
| 01 | 24/03/2026 | 21st Century AV Ltd | — | Initial Issue |